Data-Informed Adaptive Nonuniform Acquisition (DIANA): Using Bayesian Experimental Design to Optimize Nonuniform Sampling

Artificial Intelligence is playing an increasingly significant role in data processing and automated analysis of Nuclear Magnetic Resonance (NMR) experiments. Among its applications are: NUS processing; structure calculations; shaped pulse optimization; chemical shift anisotropy (CSA) modeling; chemical shift prediction.

However, thus far, ML has been used as either a preparatory method, or for managing acquired data. Thus far, to our knowledge, ML does not play a fundamental role in the process of data collection, in which the NMR spectrometer makes independent decisions for how to acquire information optimally. We propose to firstly develop and implement an algorithm enabling an NMR spectrometer to decide, in real time, which evolution time points to sample during a Nonuniformly Sampled (NUS) experiment. Briefly, NUS is a technique that aims to maintain high data quality while reducing the number of evolution time points collected. The chief advantage of this technique is a dramatic reduction in time required  to complete a multidimensional experiment, while often increasing sensitivity and resolution. Currently, a sampling schedule that encodes the evolution time points is generated by the user prior to beginning an experiment. This schedule is determined by: size of the biological system that is being studied, isotopic labeling scheme, the sample’s spin relaxation properties, and the information targeted.

Instead, develop a machine learning algorithm using Bayesian methods that learns the effect that different time points have on the sensitivity and resolution of a spectrum, as well as how to collect them in such a way as to eliminate aliasing artifacts. Without prior knowledge of the biological system, the algorithm informs the spectrometer on how to use data history to assess and adjust the experiment’s sampling schedule trajectory as the experiment is progressing. The model must understand the relationship between different evolution times and fundamental aspects of spectroscopy: sensitivity, resolution, and aliasing. In addition, the algorithm learns where the optimal stopping point will be, beyond which additional time points will provide diminishing returns on spectral quality.

Several methods exist to determine an optimal NUS sampling schedule. For example, this method is similar to Targeted Acquisition, an algorithm developed by Orekhov and coworkers to optimize the data collection process actively. However, while they similarly collect NUS data points “on the fly” and decide a stopping point based on a spectral quality factor, there is no communication between the TA and data collection processes to decide which points would be optimal.

While random sampling has been accepted as most effective, our motivation is to experiment with a different approach to spectroscopy, in which AI takes a more active role in data collection. This research is not without precedent, as a simple proof of concept showed that using variational Bayesian analysis for active NUS may be more efficient then choosing a preconceived sampling schedule1. However, this work utilized ML methods that are slightly dated, and there has been much progress in Machine Learning statistical estimation methods since.

Our findings are that MARIE performs…

We then test this method in both a Solution and Magic Angle Spinning  (MAS) NMR context, using small molecules and Ubiquitin.

Principles of NMR Acquisition

The Free Induction Decay (FID) is a voltage signal response of an isotopically-enriched sample to alternating current. The time-domain signal is rich with structural (frequency) and dynamic (decay) information, and is modeled as a linear combination of exponentially decaying sinusoids. We can model this mathematically in the following way:

st=j=1JIjeiϕ0jd=1Deidωjdtd-λjdtd

This shows the time-domain signal within a multicomplex algebra Hd.Within this context, imaginary numbers signify a 90o phase shift to account for quadrature detection in each dimension. For a one dimensional experiment, D=1 and the complex exponential behaves in a familiar manner. However, for higher dimensionality, complex numbers belonging to different dimensions do not mix. For example, in the case of two dimensions:

st1,t2=l=1LIleiϕ0ld=12eidωldtd-Rldtd=l=1JCl1cosωl1t1cosωl2t2+i1sinωl1t1cosωl2t2+i2cosωl1t1sinωl2t2+i1i2sinωl1t1sinωl2t2

Where i1i2≠ -1. As a result, each dimension increases the number of terms in the time-domain signals by a factor of 2. Overall, for a multicomplex algebra Hd, the number of terms that must be collected independently are 2D. Therefore, the number of indirect elements (Indel) increases as 2D-1.

We then extract the frequency information from this signal using the Fourier Transform (FT):

Sωk1,ωk2=1N1N2n1=0N1-1n2=0N2-1e-i12πk1n1N1+i22πk2n2N2st1,t2

The full FT for a D-dimensional experiment is expressed as the following:

Sωk1…ωkD=1N1…NDn1N1-1…ndND-1e-i12πk1n1N1+…+iD2πkDnDNDst

Translating this series to NMR, we have a signal of length TP, to which a K number of individual signals is contributing. We also include some additional terms to take into account the following: that there is a time delay in the acquisition t0, a possible phase to the initial signal ϕ0, and a relaxation decay constant λ0 (in the case of a damped harmonic oscillator) . Over all l spins being detected:

sn≔stn=lS0,l eiϕ0,leiω0,l(tn+t0)∙e-λ0,l(tn+t0)

We simplify by absorbing the initial phase term into the coefficients:

sn=lS0,l eiω0,l(tn+t0)∙e-λ0,l(tn+t0)

NMR analysis requires the discrete Fourier transform of this signal:

Sk≔Sωk=S2πkNΔdw= n=0N-1s(tn)e-iωktn=n=0N-1stne-i2πk n/N

Where

ωk=2πktacq=2πkNΔdw,     tn=nΔdw     ,  tacq=NΔdw

Importantly, this indicates that the frequencies we can observe are harmonics of the same base frequency whose period is the length of the total acquisition time (in addition to any post-processing zero-filling and linear prediction we might use (?)). Ideally, the acquisition time and number of samples are long and large enough that the detectable frequencies occur along a quasi-continuous range for increased precision (?).

Combining these expressions, we arrive at the result of the Fourier Transform:

Sωk=S0 eiω0-λ0t0+iϕ01-eNΔdw(i(ω0-ωk)-λ0)1-eΔdw(i(ω0-ωk)-λ0)

Where ω0 is the “true” frequency at which the spin is evolving (?). The value of Sωk is maximal when ω0=ωk where a peak occurs (?). In order to avoid symmetry across the carrier, both real and imaginary parts of the time-domain signal must be collected, Fourier Transformed, and added to one another. This is achieved using quadrature detection, which occurs easily in the direct dimension but requires the acquisition of two separate FIDs in the indirect dimensions to form a hypercomplex dataset, thus increasing acquisition time.

Where n is the index of the time point, while k marks the index of the frequency harmonic, ranging from -N/2 to N2-1 and k=0 marking the carrier frequency.

It is important to note that the Fourier Transform here depicts the range of possible signals, generally dependent on the sampling rate.

At the same time, the Nyquist sampling theorem reveals that the window of observable frequencies ω/2π is limited by ±12Δdw. As a consequence, for every N time points collected, we can detect N2 frequencies. Frequencies that are excited but that lie outside the spectral window will appear at an aliased frequency.

The inverse Fourier Transform is then:

sn= k=0N-1Skeiωktn,

Thus far, we have explained in simple terms the mathematical description of a 1-dimensional time signal and its Fourier Transform. However, our purpose is to use NUS for multidimensional experiments, where the descriptions and processing become more complex. Typically, there will be

st=j=1JIjeiϕ0jd=1Deidωjdtd-λjdtd

Here, id denotes the fact that we are dealing with multicomplex algebra, which we require to take quadrature detection in both the direct and indirect dimensions into account. 
sNUSt=m∘ SAB S(t1)×S(t2)

Where m is the binary mask applied to the uniformly sampled schedule, composed of ones and zeros where data points are collected and omitted, respectively.

Bayesian Regression

Thus far, most of ML applications in NMR make use of Deep Learning, in which neural networks are built to optimize models… NNs are popular due to their… Regression is a form of supervised learning that calculates a set of parameters w that form a parametric model of the provided training data.

y= Φxw+ϵ

Here, Φ(x)w is our model, where, Φ is an M×D feature map and w is a D×1 vector of weights.

Φx= - ϕx1⊺-- ϕx2⊺-… - ϕxM⊺-

An additional noise term ϵ is also included. The model is optimized by minimizing a cost function, which measures how well the model fits the training data.

Bayesian Regression takes a probabilistic approach to determining a distribution of possible parameters. Here, ϵ is the noise, which we assume to be a Gaussian distribution with mean 0 and variance σ:

ϵ ~ N0,σ

A consequence of this is that the function y also follows a Normal distribution:

ym ~ N(ϕxm⊺w, σ)

The probability of all data points can be described as the following:

pyΦ,w=m=1Mpym|ϕxm,w=m=1M12πσ2e-12σ2(ym-ϕxm⊺w)2

The Maximum Likelihood Estimate is therefore the following:

argmaxw logpyΦ,w=argmaxwm=1Mlogpym|ϕm,σ=argmaxw-N2log2πσ2-12σ2y-Φxw2=argminw y-Φxw2=argminw Lw

Solving for the weights that maximize the likelihood of the data points is therefore equivalent to the least squares L2 loss in polynomial regression. However, this treatment is important to show that we can interpret the problem in terms of probabilities, by optimizing for the set of weights that maximize the probability of observing the data given the model we have chosen. As a result, solving for the weights that minimize this expression (by taking the derivative and setting it to zero) is the following:

w=Φ⊺xΦx-1Φ⊺(x)y

The drawback of MLE however is its tendency to overfit data. We can therefore take a step further via Bayes’ Rule, which allows us to reason about things we don’t know (the relative intensities of fundamental modes in my data i.e. the Fourier spectrum coefficients) in terms of what we do know (a set of measurements for my FID):

pwy=pywpwpy

Here, pwy is the posterior distribution, which can be described by the following statement: if I know my spectrum, I know the distribution of weights present in the data.

pyw is referred to as the likelihood: if I know my spectrum, then I know what measurements I can expect. In some ways, this can be thought of as the mean of the distributions of possible inverse Fourier Transforms.

pw is the prior, which defines a reasonable range of possible spectra. The prior mathematically encodes our beliefs about the system. For example, we must take into account that the majority of the spectrum is sparse (i.e. most weights will have a value of zero).

which describes the probability that our weights will follow certain expectations. In our case, the weights are the intensities that describe the relative contribution of different frequency modes present in the FID.

Finally, py is the marginal likelihood, which provides the possible distribution of data present independent of other variables under the chosen model. It is also called the evidence as it is at times used to determine how well a model describes the system. For our purposes, it is essentially a normalizing term.

For computational efficiency, it is most convenient to start with probability relations that are conjugate with one another, leading to analytical approximations. We can do so by imposing a normal distribution over the prior, with mean 0 to encode sparsity. This is especially accurate for some kinds of spectroscopy in which both positive and negative intensity values are present, such as double-quantum correlation experiments, though most spectra are restricted to positive values.

If we assume a Gaussian prior over the weights2 (thus leading to what is called Ridge Regression) of mean 0 and standard deviation χ, then:

p(w)=d=1Dp(wd)=d=1D12πχ2e- 12χ2 wd2

Our approximation of the weights, called the Maximum A Posteriori Estimate, is then the following:

wMAP= argmaxw logpwy,x=argmaxw logp(y|w,x)+logp(w)=argmaxw -12σ2y-Φxw2-12χ2w2=argminw y-Φxw2+σ2χ2w2=argminw y-Φxw2+λw2=argminw Lw

Where lambda is a regularization parameter that can be tuned in order to balance the tradeoff between minimizing the weights w, and the Maximum Likelihood Estimate (or linear regression) term. Minimizing this loss function with respect to w results in the following:
wMAP=wprior+Φ⊺xΦx+λΙ-1Φ⊺xy

=Φ⊺xΦx+λΙ-1Φ⊺xy= Σ∙Φ⊺xy

Since the prior mean is 0. Here, Σ is the posterior variance.

In practice, using a gaussian prior over the weights is not the most realistic in terms of our expectations of what a spectrum typically looks like. However, conjugate relationships between the prior, likelihood, and marginal likelihood render the posterior relatively simple to compute in closed-form, or else resort to more complex Variational Bayesian methods.

Choosing New Data Points

If two random variables are jointly Gaussian (a multivariate normal distribution), then their combination (?) can be modeled as:

xy~Nab,ACC⊺B=Nab,ΣoldCC⊺Σu

As a result, the conditional distribution

y|x ~Nb+C⊺A-1x-a, B-C⊺A-1C= Nμ,Σu-C⊺Σold-1C

So that the variance of the conditional is:

Σ =Σu-C⊺Σold-1C

We write the posterior variance of a hypothetical new dataset, in which a single new point is chosen from the unsampled points and added to the list of datapoints:

S⋆=Φx⋆⊺Φ(x)⋆+λΙ-1

The dimensionality of S⋆ is the number of previously collected points, plus one.

The covariance of the posterior predictive is then:

Σpred=Φ(x)⋆S⋆Φx⋆⊺+σ2Ι

We then wish to decompose this predictive covariance in order to isolate the matrix components that correspond to the theoretically sampled point:

Σpred=ΣoldCC⊺Σu

Materials and Methods

Synthetic spectra were generated using an in-house python script as described previously, using a database that is publicly available on the MARIE website. In short, we created a synthetic spectral database by generating peptides with a random number of each fundamental amino acid, and then generating a random chemical shift within a standard deviation of averages provided in the BMRB.

Quality Factor for Spectra:

There are different ways to assess quality in spectra. Previous work has used chemical shift precision, intensity, and expected number of peaks3. This may be useful when training a model using synthetic spectra in the case of supervised learning. However, we will begin with a different approach, of entropy minimization: using the data that has been collected thus far, we wish to sample next in the areas where the greatest uncertainty in the spectral coefficients exists.

If we define a quality metric that averages over possible spectra:

QN=dS1…dSk QS1,…Sk pS1,…,Sky1,…yN

We can define the Entropy as:

HS1,…Sky1,…,yk=-dS1…dSkpS1,…,Sky1,…yNlogpS1,…,Sky1,…yN

For a multivariate Gaussian, the expression reduces to:

HS1,…Sky1,…,yk=K21+ log2π+ 12logΣ

Here, Σ is the covariance of the posterior likelihood.

Concept:

We begin by expressing a single time point as the inner product of a vector of “weights” S and a vector of features ϕtn, as well as a noise variable εn that follows a Gaussian normal distribution of mean 0 and variance σ2:

yn=S⊺ϕtn+εn,  εn ~ Ncx0,σ2

Here, we slightly adjust the notation for the normal distribution, Ncx, to indicate that the random variable is complex, so that there are in fact two normal distributions corresponding to the real and imaginary part of the variable. The vector of “weights” S indicates the relative presence of the different “harmonics” (or frequencies) present during the point yn while ϕtn is a vector of possible frequencies present in the signal:

S=S0…SN?-1, ϕtn=eiω0tn…eiωN?-1tn

In this way, the vector S represents the spectrum in one dimension, and our goal is to accurately

If we initiate the experiment by collecting a series of time points n1, we can apply Bayes’ Rule in the following way:

pSyn1=pyn1Sp(S)p(yn1)

Here, p(S) is the prior distribution over S, meaning the probability that the selected weights S are present in the data. pyn1S is the likelihood, or the probability of observing data points yn1 given the initiating weights S.  p(yn1) is the marginal likelihood, which represents the probability of the initial measurements (?). pSyn1 is the posterior distribution, which relates the uncertainty over our understanding of S based on the data collected.  We define the following:

pS=k=0N-1p(Sk),  Sk=Ncx0,χk2

and

pyn1S=NcxS⊺ϕtn,σ

Once we have calculated the posterior distribution pSyn1, we can update our weights and the prior distribution:

pSyn2=pyn2SpSyn1p(yn2)

Where n1 is a subset of the new measurement set n2.

BED Algorithm:

Acquisition ={  }

for t⋆ in set of unevaluated points:

S ~ pSy1,…,yN

s=iFFT(S)

ϵ⋆= N0,σ

y⋆=st⋆+ϵ⋆

Q⋆=dS1…dSk QS1,…Sk pS1,…,Sky1,…yN,y⋆

Acquisition{t⋆}=ImprovementQ⋆,QN

(1) Worley, B. Variational Bayesian analysis of nonuniformly sampled NMR data. Concepts in Magnetic Resonance Part A 2017, 46A (2), e21428. DOI: https://doi.org/10.1002/cmr.a.21428.

(2) O'Hagan, A. Bayes–Hermite quadrature. Journal of Statistical Planning and Inference 1991, 29 (3), 245-260. DOI: https://doi.org/10.1016/0378-3758(91)90002-V.

(3) Jaravine, V. A.; Orekhov, V. Y. Targeted Acquisition for Real-Time NMR Spectroscopy. J. Am. Chem. Soc. 2006, 128 (41), 13421-13426. DOI: 10.1021/ja062146p.